Lab: Find the intrinsic and extrinsic matrix for L515

This folder has the following files

- L515-calibration-class.ipynb – use to acquire data from L515, calibrate camera, find intrinsic and extrinsic matrix

- Sample images of checkerboard

- Checkerboard-A4-28mm-9x6.svg

Task -1 Stream L515 from sdk

- Configure L515 lidar-camera

Follow the link  - https://github.com/IntelRealSense/librealsense/releases/tag/v2.54.2

Install: https://github.com/IntelRealSense/librealsense/releases/tag/v2.54.2  (Install Intel.RealSense.SDK-WIN10-2.54.2.5684.exe)

- Connect the L515 to laptop

- Using the real sense viewer make sure that the camera is streaming

- Close real sense viewer

Python wrapper for L515:

- Home page: https://github.com/IntelRealSense/librealsense/tree/development/wrappers/python

- Install pyrealsense2: pip install pyrealsense2==2.54.2.5684 (https://pypi.org/project/pyrealsense2/ ), https://pypi.org/project/pyrealsense2/2.54.2.5684/ q

- Tutorials: https://github.com/IntelRealSense/librealsense/tree/development/wrappers/python/examples

- Pyrealsense API: https://intelrealsense.github.io/librealsense/python_docs/_generated/pyrealsense2.html 
Task – 2 Calibrate L515 camera

- Print the chess pattern – Checkerboard-A4-28mm-9x6.svg

- Measure the approximate distance of checkerboard from camera lens (this is z_c)

- Run L515-calibration-class.ipynb (cell by cell and follow the instructions given in each cell)

- Run MAS512_2023_AUTUMN virtual environment

  - Capture image

    - Focus the camera to the chess pattern so that entire board is visible in the frame

    - Press S to capture the image

    - Rotate the camera and capture another image by pressing S

    - The images will be named with timestamp

    - Take at least 15 -20 images

    - The images will be saved in /captured_images. This folder will have RBG and depth images

    - Copy RGB images in directory where Run L515-calibration-class.ipynb is located

- Intrinsic calibration

  - Run Run L515-calibration-class.ipynb cell wise cell

  - This gives you the intrinsic matrix (mtx)

  - This program also saves the intrinsic matrix as B. npz

- Extrinsic calibration

  - Run  L515-calibration-class.ipynb cell wise cell

  - This program gives you rotational (rvec) and translational matrix (tvec)

  - Append rvec and tvec horizontally to get extrinsic matrix